# Hardware Design and Component Specification for an ESP32-Based Multi-Channel 2.4 GHz and Sub-GHz RF Spectrum Analyzer and Jammer Platform

## Architectural Rationale and System Concept

Modern wireless security auditing and spectral monitoring require highly integrated, portable hardware architectures capable of analyzing and interacting with diverse radio frequency (RF) protocols1. Developing a multi-protocol diagnostic platform involves utilizing an Espressif ESP32 microcontroller as the central computational unit, complemented by an array of specialized RF transceivers2. The primary objective of such a design is to bridge the gap between high-frequency digital signal processing and localized RF physical-layer manipulation1.

To achieve multi-band coverage across the 2.4 GHz Industrial, Scientific, and Medical (ISM) band and various sub-GHz frequencies, the system architecture employs a modular transceiver array1. A single transceiver module is physically limited by its frequency synthesizer, which can only lock onto and monitor one frequency channel at any given instant7. To overcome this hardware limitation and enable simultaneous multi-channel sniffing, signal analysis, and active defensive signal generation, the platform integrates multiple Nordic Semiconductor nRF24L01+ modules alongside a Texas Instruments CC1101 sub-GHz transceiver1.

This multi-module configuration allows the system to execute parallel tasks, such as dedicated spectrum sweeping on one module while simultaneously performing targeted packet sniffing or signal disruption on others1. The ESP32 is highly suited for coordinating this array due to its dual-core Xtensa processor architecture, which allows developers to decouple timing-critical SPI RF driver routines on one core from user-interface rendering, file system access, and sensor logging on the other9.

## Technical Specifications and Component Allocation

Executing a professional-grade hardware design requires a detailed, verified Bill of Materials (BOM)11. The selected components must balance electrical compatibility, thermal limits, and physical footprint requirements7. The following table compiles the exact hardware components, passive filtering elements, and expansion sensors necessary to construct a fully integrated spectrum analyzer and testing platform9.

| Component Category | Description / Manufacturer Part | Quantity | Operational Role | Key Technical Specifications and Selection Criteria |
| --- | --- | --- | --- | --- |
| Microcontroller | Espressif ESP32-WROOM-32U11 | 1 | Central system processing, GUI rendering, and native Wi-Fi/BLE auditing9. | Dual-core processor2; features an integrated U.FL connector for external high-gain antennas11. |
| RF Transceiver (2.4 GHz) | Nordic nRF24L01+PA+LNA5 | 3 or 41 | 2.4 GHz spectrum scanning, packet sniffing, and signal disruption1. | Features a logic-controlled power amplifier and low-noise amplifier; peak output5. |
| RF Transceiver (Sub-GHz) | Texas Instruments CC1101 Breakout4 | 1 | Sub-GHz protocol analysis, signal replay, and IoT monitoring2. | Operates across 315, 433, 868, and 915 MHz; support for configurable FSK/GFSK/OOK modulation7. |
| Display Module | ILI9341 2.4-inch TFT LCD with SPI Touch5 | 1 | Real-time spectrum visualization and user menu navigation5. | SPI-controlled active matrix; includes an on-board MicroSD slot sharing SPI lines9. |
| Power Converter | MP1584EN Synchronous Step-Down12 | 1 | High-efficiency primary step-down from battery/USB to 12. | Supports input voltages up to ; delivers up to with efficiency exceeding 12. |
| Linear Regulator | MIC5219-3.3 Low-Dropout Linear Regulator11 | 2 | Clean, low-noise rail dedicated to the sensitive RF modules12. | Ultra-low-noise output with capability; isolates RF transceivers from digital logic noise12. |
| Battery Management | TP4056 Linear Li-Ion Charger with Protection9 | 1 | Safe linear charging and over-discharge protection for 18650 cells9. | Features dual LED status indicators and built-in protection against cell over-discharge below 11. |
| Storage Module | SPI-based MicroSD Card Slot4 | 1 | Local logging of captured signal packets, PCAP files, and configuration data4. | Interface wired to share hardware SPI with the display, requiring strict tri-state buffer control18. |
| Geospatial Sensor | Neo-6M GPS Module9 | 1 | Real-time coordinate capture for spatial mapping of spectrum data9. | Communicates via hardware UART; tracks up to 50 satellite channels with a update rate9. |
| Environmental Sensor | BME280 Temperature, Humidity & Pressure15 | 1 | Atmospheric data logging during long-term field auditing11. | Communicates over I2C; low current draw of at sampling frequency11. |
| Voltage Sensor | MAX17043 LiPo Fuel Gauge | 1 | High-precision battery voltage and state-of-charge tracking | Communicates over I2C; uses a ModelGauge algorithm to eliminate the need for sense resistors. |
| Optical Sensor | TEMT6000 Ambient Light Sensor | 1 | Automatic screen backlight brightness adjustment to optimize power | Analog output mapped to an ESP32 ADC pin; peak sensitivity matches the human eye. |
| Visual Indicator | WS2812B NeoPixel RGB LED1 | 1 | Status and mode signaling via configurable color outputs1 | Single-wire digital control; cascades easily and draws minimal power during idle states11. |
| User Inputs | Tactile Pushbuttons11 | 5 | Physical menu navigation (Up, Down, Left, Right, Select)11 | Interfaced to GPIOs configured with internal pull-up resistors and software debounce11. |
| Passive Filters | Ceramic Decoupling Capacitors12 | Multiple | High-frequency noise suppression close to transceiver VCC pins12. | and multi-layer ceramic capacitors (MLCC) with low equivalent series resistance5. |
| Passive Filters | Bulk Electrolytic Capacitors12 | 4 | Buffering power draw during high-gain transmission transients12. | low-ESR radial electrolytic capacitors rated to at least 24. |

## Circuit Schematic and Pin Mapping

To ensure proper communication and prevent pin multiplexing conflicts, the physical connections must be planned in accordance with the ESP32’s internal hardware routing capabilities10. The display and storage peripherals are mapped to the VSPI hardware bus, while the RF transceivers utilize the HSPI bus26. The following table provides the precise pin mappings for the entire system, establishing an unambiguous wiring schematic5.

| Peripheral Module | Physical Pin on Module | ESP32 GPIO Destination | Signal Designation | Hardware Interface Details and Configuration Notes |
| --- | --- | --- | --- | --- |
| nRF24L01+ Module #1 [cite: 27] | VCC16 | Dedicated LDO Output | RF | Powered from the clean, isolated analog regulator rail12. |
|  | GND16 | Common Ground | GND | Connected directly to the system-wide ground plane12. |
|  | SCK16 | GPIO 1427 | HSPI_SCLK26 | Dedicated hardware clock line for RF operations26. |
|  | MOSI16 | GPIO 2327 | HSPI_MOSI26 | Master Out Slave In line routing data to transceivers26. |
|  | MISO16 | GPIO 1927 | HSPI_MISO26 | Master In Slave Out line capturing data from transceivers26. |
|  | CE16 | GPIO 523 | RF1_CE23 | Configured as a digital output to enable module RX/TX8. |
|  | CSN16 | GPIO 1723 | RF1_CSN23 | Active-low Chip Select; kept HIGH during idle states8. |
| nRF24L01+ Module #2 [cite: 27] | CE | GPIO 1627 | RF2_CE | Configured as a digital output to enable module RX/TX8. |
|  | CSN | GPIO 427 | RF2_CSN | Active-low Chip Select; kept HIGH during idle states8. |
| nRF24L01+ Module #3 [cite: 27] | CE | GPIO 1527 | RF3_CE | Configured as a digital output to enable module RX/TX8. |
|  | CSN | GPIO 227 | RF3_CSN | Active-low Chip Select; kept HIGH during idle states8. |
| CC1101 Sub-GHz Transceiver [cite: 4] | CSN17 | GPIO 21 | CC_CSN | Active-low Chip Select isolating Sub-GHz register writes17. |
|  | GDO017 | GPIO 22 | CC_GDO0 | Interrupt pin indicating payload receipt or TX complete17. |
| ILI9341 TFT Display [cite: 5, 9] | SCLK | GPIO 18 | VSPI_SCLK26 | Shared high-speed clock for display rendering26. |
|  | MOSI | GPIO 235 | VSPI_MOSI26 | Shared data line routing frame data to the screen26. |
|  | MISO | GPIO 195 | VSPI_MISO26 | Shared data return line; active during register reads26. |
|  | CS5 | GPIO 155 | TFT_CS | Active-low line enabling SPI communication with display5. |
|  | D/C | GPIO 21 | TFT_DC | Data/Command selection line for display register control. |
|  | RST | GPIO 22 | TFT_RST | Hardware reset pin to initialize display controller on boot. |
| MicroSD Card Module [cite: 4, 11] | CS | GPIO 13 | SD_CS | Active-low Chip Select; requires MISO isolation18. |
| Neo-6M GPS Module [cite: 9] | TX | GPIO 34 | GPS_RX | Hardware UART receiver channel mapped to ESP32 ADC/Input. |
|  | RX | GPIO 35 | GPS_TX | Hardware UART transmitter channel. |
| Environmental Sensor (BME280) | SDA | GPIO 21 | I2C_SDA27 | Hardware I2C data line; requires pull-up resistors11. |
|  | SCL | GPIO 22 | I2C_SCL27 | Hardware I2C clock line; requires pull-up resistors11. |
| Tactile Buttons (5-way array) | UP27 | GPIO 2627 | BTN_UP27 | Pin configured as input with internal pull-up resistor11. |
|  | SELECT27 | GPIO 3227 | BTN_SEL | Pin configured as input with internal pull-up resistor11. |
|  | DOWN27 | GPIO 3327 | BTN_DOWN27 | Pin configured as input with internal pull-up resistor11. |
|  | LEFT | GPIO 2527 | BTN_LEFT | Pin configured as input with internal pull-up resistor11. |
|  | RIGHT | GPIO 2727 | BTN_RIGHT | Pin configured as input with internal pull-up resistor11. |
| NeoPixel WS2812B [cite: 1, 11] | DIN | GPIO 12 | WS2812_DATA | Single-wire high-speed timing-sensitive data interface11. |
| Ambient Light Sensor | OUT | GPIO 36 | ALS_ANALOG | Analog output read via internal 12-bit ADC Channel 1. |

## RF Signal Analysis and Jamming Mechanisms

The primary operation of the diagnostic tool is split between passive signal analysis and active transmission testing, which can function as a jammer for vulnerability research1. The underlying physics of these operations dictate distinct software execution models and strict transceiver state transitions17.

### Spectrum Analysis Mechanics

To construct a real-time visualization of the 2.4 GHz spectrum, the platform leverages the nRF24L01+ Received Power Detector (RPD) register29. Register 0x09 contains a single-bit field at index 0 that reflects whether the power level in the currently selected channel exceeds the hardware threshold of 16. The scanning sweep routine is executed sequentially across 128 channels, spaced at  increments starting from  up to 8.

For each channel step, the firmware writes the channel offset to the RF_CH register (0x05), toggles the Chip Enable (CE) pin high for approximately  to allow the phase-locked loop (PLL) frequency synthesizer to lock and stabilize, and then reads the RPD register8. If the RPD bit is returned as high, a signal level exceeding  is present, and the hit counter for that specific channel increment is updated in the display frame buffer30.

Integrating multiple nRF24L01+ modules allows the sweep workload to be divided1. Assigning Module 1 to sweep channels 0–42, Module 2 to sweep channels 43–85, and Module 3 to sweep channels 86–127 reduces the total scan period by a factor of three, resulting in high-speed, real-time waterfall updates on the UI display1.

### Active Jamming Methodologies

When configured in active transmission mode, the system acts as a signal jammer to test the robustness of wireless protocols1. Physical layer jamming can be achieved through two distinct approaches1:

- Constant Carrier Wave Injection: The transceiver is programmed into a non-standard test state where the PLL frequency synthesizer is locked onto a target channel while packet framing, preamble generation, and cyclic redundancy checks (CRC) are bypassed17. This forces the module to transmit a continuous, unmodulated RF carrier wave at maximum power () on the target channel, raising the noise floor and preventing nearby receivers from decoding valid packets16.

- Pseudo-Random Frequency Hopping Disruption: Modern protocols like Bluetooth and Wi-Fi utilize Frequency Hopping Spread Spectrum (FHSS) or Orthogonal Frequency Division Multiplexing (OFDM) to avoid static channel noise7. To disrupt these agile signals, the ESP32 orchestrates the multi-transceiver array to sweep rapidly across a wide frequency band1. Module 1 can be configured to continuously flood BLE advertising channels (37, 38, and 39) with random, invalid packet headers1. Simultaneously, Modules 2 and 3 can be commanded to sweep pseudo-randomly across Wi-Fi channel bands, injecting high-rate noise payloads to trigger packet collisions and force continuous retransmission attempts by target devices2.

## Electromagnetic Compatibility, Power Filtering, and SPI Bus Coexistence

Combining high-frequency digital microcontrollers, multi-layer liquid crystal displays, high-power RF transmitters, and localized flash memory onto a single development platform introduces significant signal integrity issues9. Managing these issues requires addressing power supply noise and SPI bus contention18.

### Power Supply Noise and Thermal Dissipation

The current consumption profile of the platform is highly dynamic9. While the ESP32 averages approximately  during processing, it can spike to  when transmitting native Wi-Fi beacons3. Simultaneously, each nRF24L01+PA+LNA transceiver draws up to  during peak power packet transmissions16.

Under worst-case scenarios where three nRF24 modules and the sub-GHz CC1101 transmit simultaneously alongside the active TFT display backlight and high-speed SD card writes, the aggregate peak load is calculated as7:

To step down an input voltage of  (typical of standard external wall adapters) to the required  system rail, a simple linear regulator would be highly inefficient12. The thermal power dissipation () of a linear regulator is expressed as37:

Given that a standard SOT-223 regulator has a junction-to-ambient thermal resistance () of approximately  without extensive heat sinks, a power dissipation of over  would instantly trigger thermal shutdown or cause hardware failure35.

To resolve this, the design utilizes a dual-stage regulation topology12. A synchronous buck converter (MP1584EN) is placed at the primary input stage to step down the external voltage ( to ) to an intermediate  rail with up to  conversion efficiency12. This intermediate  rail then feeds two independent, low-noise LDO linear regulators11. The first LDO regulates  for the digital logic, display, and SD card12. The second LDO is dedicated entirely to the sensitive analog RF transceivers12. This isolates high-frequency digital noise and preventing voltage drops during SD card writes from corrupting the RF ground plane17.

### SPI Bus Contention and Level Shifter Lockup

Integrating multiple high-speed SPI devices onto the ESP32’s hardware buses can lead to bus lockups10. A common point of failure involves the level-shifter integrated circuits (such as the 74HC125) typically included on cheap MicroSD card breakout modules18. These breakout modules are designed to convert  microcontroller signals to the  required by the SD card18. However, many of these boards feature an architectural flaw: the active-low Output Enable () pin of the level-shifter chip is tied directly to the ground plane18.

This causes the level-shifter's output buffers to drive the shared Master In Slave Out (MISO) line continuously, even when the SD card’s Chip Select (CS) line is pulled HIGH to deselect the module18. This prevents other peripherals on the shared SPI bus (such as the nRF24 transceivers) from tri-stating their MISO outputs and communicating with the ESP3218.

To prevent this MISO lockup, two hardware solutions can be implemented18:

- Physical Circuit Modification: Modifying the MicroSD card breakout module involves scraping the PCB trace connecting the level-shifter’s pin 1 and pin 4 ( buffers) to ground, and soldering a jumper wire directly from those pins to the SD card Chip Select line21. This ensures that when the SD CS line is pulled high to deselect the card, the level-shifter buffers enter a high-impedance tri-state, freeing the shared MISO line18.

- Dedicated Software SPI Isolation: Rather than placing the SD card and the display on the same hardware SPI bus as the sensitive transceivers, the storage card can be isolated onto separate GPIO pins using the SdFat library’s software SPI (SoftSPI) implementation18. Since diagnostic logging is typically less latency-sensitive than RF packet capture, dedicating the ESP32’s primary hardware HSPI and VSPI buses to the display and the transceiver array optimizes signal timing10.

## Regulatory Compliance and Legal Frameworks

The operation of RF-emitting apparatus, particularly devices configured for signal analysis and active transmission testing, is heavily governed by national and international telecommunications laws39. Operating unlicensed transmitters or emitting active interference can result in severe legal consequences39.

### Telecommunications Regulatory Commission of Sri Lanka (TRCSL) Framework

In Sri Lanka, the Telecommunications Regulatory Commission (TRCSL) is the statutory body responsible for spectrum management and licensing under the Sri Lanka Telecommunications Act No. 25 of 199140.

The TRCSL enforces several strict regulatory compliance mandates:

- Importation and Possession Bans: Under Section 36 of the consolidated telecommunications statutes, it is a criminal offence to import, possess, or operate any wireless telegraphy apparatus or terminal equipment without an authorized licence of the prescribed description issued directly by the regulatory authority41.

- Type Approval Requirements: Under Section 5 of the Act, all Radio and Telecommunications Terminal Equipment (RTTE) must undergo a formal Type Approval process before importation, sale, or commercial deployment40. Standard Type Approval requires submitting laboratory test reports from accredited institutions to verify that the equipment complies with electromagnetic compatibility (EMC), radio frequency (RF) exposure, and health standards45.

- Low-Power Device Exemptions: The TRCSL provides exemptions from individual station licensing for certain low-power, short-range devices (SRDs)40. However, these devices must operate strictly within designated bands and must not exceed specified Equivalent Isotropically Radiated Power (EIRP) limits, typically set at  () for LPWAN bands or under  for standard short-range Bluetooth and Wi-Fi devices40.

### Enforcement, Penalties, and Spectrum Surveillance

The TRCSL operates a network of fixed and mobile spectrum monitoring and direction-finding (DF) stations across Sri Lanka, capable of monitoring frequencies from  to 39. These stations are designed to identify, isolate, and locate unauthorized radio emissions, illegal transmitters, and active signal jammers39.

Under Section 39 of the Sri Lanka Telecommunications (Amendment) Act No. 39 of 2024 and consolidated regulatory statutes, the legal penalties for unlicensed operation and spectrum interference are severe43:

- Fines and Imprisonment: Any individual or operator who installs, possesses, or works an unlicensed radio transmitter or causes active, harmful interference to licensed commercial, emergency, or military communications networks commits an offence39. Upon conviction by a Magistrate, the offender is liable to a fine not exceeding ten million rupees, imprisonment for a term not exceeding one year, or both47.

- Asset Seizure: Regulatory authorities, supported by law enforcement and the Ministry of Defence, have the power to enter properties, seize illegal transmitters, and permanently confiscate all associated computer systems, power equipment, and antenna structures41.

- Active Jamming Prohibition: Active signal jamming devices (which block cellular networks, GPS, Wi-Fi, or emergency channels) are strictly illegal under civil frameworks worldwide1. Because these devices can disrupt critical infrastructure and emergency services, their deployment is classified as a severe threat to public safety and national security39.

Therefore, any experimental verification of the RF transmission, packet injection, or signal disruption mechanisms described in this design must be conducted strictly within a shielded laboratory environment1. This includes the use of sealed Faraday cages or calibrated RF anechoic chambers to ensure that no signals escape into the public spectrum, thereby maintaining compliance with national and international laws39.

## Architectural Synthesis and Implementation Summary

To assist developers in avoiding common hardware integration issues, the final system assembly should align with the structural recommendations consolidated in the following table2.

| Design Challenge | Primary Cause | Mitigation Strategy and Architectural Implementation |
| --- | --- | --- |
| RF Transmit Failure [cite: 24, 25] | Voltage drops under transient load peaks during transmission12. | Solder a low-ESR ceramic and electrolytic capacitor directly to the VCC/GND pins of each transceiver5. |
| Self-Interference / EMI [cite: 25, 52] | High-frequency noise from unshielded PA/LNA transceivers25. | Utilize metal shield cans on RF modules and route the ESP32’s on-board antenna away from transceiver antennas7. |
| SPI Bus Freeze [cite: 18, 21] | Level-shifter output conflict on cheap SD card breakout boards18. | Modify the card slot PCB to connect level-shifter enable pins directly to Chip Select, or isolate storage using Software SPI18. |
| Regulator Overheating [cite: 12, 37] | Excessive power dissipation dropping to via linear regulation12. | Implement a dual-stage system using a switching buck converter for primary step-down, followed by low-noise linear regulators12. |
| Regulatory Liability [cite: 39, 41] | Unauthorized signal emissions violating local laws39. | Disable all active signal generation software modules during open-air testing; conduct all active transmission trials in a Faraday cage1. |

#### Works cited

- wifi-jammer · GitHub Topics, https://github.com/topics/wifi-jammer

- GitHub - cifertech/nRFBox: Open-source ESP32-powered tool to scan, jam, spoof, and master BLE, Wi-Fi, and 2.4GHz networks., https://github.com/cifertech/nrfbox

- esp32 · GitHub Topics, https://github.com/topics/esp32

- Sacred Labs FlipMods Ultra V3 ESP32/NRF24/CC1101 3-in-1 WiFi Dev Board for Flipper Zero GPIO - Instruction Manual, https://manuals.plus/asin/B0DFX6Z4J1

- GitHub - arinouri/ESP32_Marauder-NRF24L01_-_TFT_Display: Successfully flashed the JustCallMeKoko/ESP32-Marauder firmware to my ESP32 and added support for an NRF24L01+PA+LNA long-range antenna., https://github.com/arinouri/ESP32_Marauder-NRF24L01_-_TFT_Display

- NRF24 ESP32 Development Board 2.4G High Gain CC1101 Support NRF Sniffer Mousejacker for Flipper Zeros, https://www.esp32s.com/product/for-flipper-zero-modification-module-nrf24esp32-development-board-2-4g-high-gain-cc1101-support-nrf-sniffer-mousejacker/

- CC1101 vs nRF24L01: Which RF Module to Choose? - Flywing Tech, https://www.flywing-tech.com/blog/cc1101-vs-nrf24l01/

- How nRF24L01+ Module Works and Interfacing it with Arduino - Play with Circuit, https://playwithcircuit.com/nrf24l01-module-arduino-wireless-communication-tutorial/

- ESP32 Marauder v4 (OG) Wiring Guide Part1 - YouTube, https://www.youtube.com/watch?v=kL4E8a-YjZw

- SPI Master driver - - — ESP-IDF Programming Guide release-v4.0 documentation, https://docs.espressif.com/projects/esp-idf/en/release-v4.0/api-reference/peripherals/spi_master.html

- Nrfbox v2 Components Bom | PDF | Printed Circuit Board | Usb - Scribd, https://www.scribd.com/document/1000748964/nrfbox-v2-components-bom

- Designing a Stable 3.3V/5V Power Supply for Microcontrollers Using Modern Voltage Regulator - Fly-Wing - Flywing Tech, https://www.flywing-tech.com/blog/designing-a-stable-3-3v-5v-power-supply-for-microcontrollers-using-modern-voltage-regulator/

- nRFBox v2.7.2: ESP32-Powered Tool to Scan, Jam & Spoof BLE, Wi-Fi & 2.4GHz Signals, https://cifertech.net/nrfbox-v2-7-2-esp32-powered-tool-to-scan-jam-spoof-ble-wi-fi-2-4ghz-signals/

- wifi-jammer · GitHub Topics, https://github.com/topics/wifi-jammer?l=c%2B%2B

- T-Embed CC1101 Plus - LILYGO, https://lilygo.cc/en-us/products/t-embed-cc1101-plus

- NRF24L01 with Arduino, NRF24L01 Pinout, and Example Codes - Electronic Clinic, https://www.electroniclinic.com/nrf24l01-with-arduino-nrf24l01-pinout-and-example-codes/

- CC1101 868MHz SPI RF Module – Complete Guide - - StudioPieters.nl, https://www.studiopieters.nl/cc1101-868mhz-spi-rf-module-complete-guide/

- Using nRF24 and SD card module together - Arduino Forum, https://forum.arduino.cc/t/using-nrf24-and-sd-card-module-together/277969

- AMS1117 3.3V DC-DC Step Down Power Supply Module | Voltage Regulator Board, https://infyniqs.com/product/ams1117-3-3v-dc-dc-step-down-power-supply-module/

- LDO Regulator vs Buck Converter: Efficiency Comparison - Zbotic, https://zbotic.in/ldo-regulator-vs-buck-converter-efficiency-comparison/

- [Question] NRF24L01+ fails when using with MicroSD card. · Issue #692 · nRF24/RF24, https://github.com/nRF24/RF24/issues/692

- nrf24l01 · GitHub Topics, https://github.com/topics/nrf24l01?l=c%2B%2B

- nRFBOX: Your All-in-One Gadget for BLE and 2.4GHz Networks - CiferTech, https://cifertech.net/nrfbox-your-all-in-one-gadget-for-ble-and-2-4ghz-networks/

- nRF24L01+ PA LNA - Seems that the PA (power amplification) portion of the module is failing? - Electronics-Lab, https://www.electronics-lab.com/forums/threads/nrf24l01-pa-lna-seems-that-the-pa-power-amplification-portion-of-the-module-is-failing.280466/

- nRF24L01+ PA LNA - Seems that the PA (power amplification) portion of the module is failing? | All About Circuits, https://forum.allaboutcircuits.com/threads/nrf24l01-pa-lna-seems-that-the-pa-power-amplification-portion-of-the-module-is-failing.127403/

- ESP32 SPI Communication: Pins, Multiple SPI, Peripherals (Arduino) | Random Nerd Tutorials, https://randomnerdtutorials.com/esp32-spi-communication-arduino/

- wfhstudio/DIY-nRF24-cifertech - GitHub, https://github.com/wfhstudio/DIY-nRF24-cifertech

- Using nrf24l01+ with other devices together on one SPI-Bus. | MySensors Forum, https://forum.mysensors.org/topic/1961/using-nrf24l01-with-other-devices-together-on-one-spi-bus

- nRF-BOX - 2.4GHz scanner/jammer/Channel Analyzer - CiferTech, https://cifertech.net/nrf-box-2-4ghz-scanner-jammer-channel-analyzer/

- Getting Started with nRF24L01+ on Arduino - maniacbug - WordPress.com, https://maniacbug.wordpress.com/2011/11/02/getting-started-rf24/

- 24ghz · GitHub Topics, https://github.com/topics/24ghz

- DIY Bluetooth Jammer Using ESP32 & NRF24L01 #jammers #diy - YouTube, https://www.youtube.com/shorts/l8_TmGZ8dwE

- wireless-jammer · GitHub Topics, https://github.com/topics/wireless-jammer

- Using Multiple SPI Devices - MicroSD Card Reader + NRF24L01 : r/arduino - Reddit, https://www.reddit.com/r/arduino/comments/kgzztj/using_multiple_spi_devices_microsd_card_reader/

- Powering ESP32 with 7.4V battery. PCB Design review - All About Circuits Forum, https://forum.allaboutcircuits.com/threads/powering-esp32-with-7-4v-battery-pcb-design-review.205137/

- AMS1117 3.3V Power Supply Module - Himalayan Solution, https://test.himalayansolution.com/product/ams1117-3-3v-power-supply-module

- AMS1117 LDO Regulator: A Deep Practical Design Guide - MOZ Electronics, https://mozelectronics.com/tutorials/ams1117-voltage-regulator-datasheets-pinout-applications/

- SPI Protocol – Serial Peripheral Interface Complete Guide - EcrioniX, https://ecrionix.org/serial-protocols/spi/

- Spectrum Monitoring - Telecommunications Regulatory Commission, https://www.trc.gov.lk/spectrum/pages_e.php?id=114

- Low Power Device - Telecommunications Regulatory Commission, http://www.trc.gov.lk/licensing/pages_e.php?id=68

- TELECOMMUNICATIONS - Lanka Law, https://lankalaw.net/wp-content/uploads/2025/02/1981Y16V521C-1.html

- Amateur Services - Telecommunications Regulatory Commission, https://www.trc.gov.lk/licensing/pages_e.php?id=60

- Sri Lanka Telecommunications, https://lankalaw.net/wp-content/uploads/2025/02/1991Y0V0C25A.html

- Type Approval - Telecommunications Regulatory Commission, http://www.trc.gov.lk/spectrum/pages_e.php?id=115

- Sri Lanka - TRCSL Type Approval | NanoTech Solutions, https://www.nanotechsol.com/srilanka-trcsl-type-approval

- Type Approval for Sri Lanka (TRCSL Certification) - DataX International, https://dataxinternational.com/type-approval/sri-lanka/

- PARLIAMENT OF THE DEMOCRATIC SOCIALIST REPUBLIC OF SRI LANKA - Telecommunications Regulatory Commission, https://www.trc.gov.lk/content/files/acts/39-2024_E.pdf

- Approval for Mobile Phones / Devices - Telecommunications Regulatory Commission, https://www.trc.gov.lk/pages_e.php?id=63

- Performance Report 2019 - The Parliament of Sri Lanka, https://www.parliament.lk/uploads/documents/paperspresented/performance-report-ministry-of-defence-2019.pdf

- performance-report-ministry-of-defence-2016.pdf - The Parliament of Sri Lanka, https://www.parliament.lk/uploads/documents/paperspresented/performance-report-ministry-of-defence-2016.pdf

- Frequency Licensing - Telecommunications Regulatory Commission, https://www.trc.gov.lk/spectrum/pages_e.php?id=113

- Will a NRF24L01+PA+LNA help with my signal problems - MySensors Forum, https://forum.mysensors.org/topic/9138/will-a-nrf24l01-pa-lna-help-with-my-signal-problems